Wireless Sensor Networks (WSNs) are used to monitor and control systems in the industrial area such as monitoring applications, radiation checks, leakage detection, distributed, and process control applications and have some advantages such as low cost and high flexibility when compared with the use of wired networks [1]. Industrial low-power wireless mesh network applications have strong requirements in terms of latency, energy efficiency, and reliability. The routing protocol is crucially important to reduce latency, energy efficiency, and reliability, so we propose a new mechanism namely Burstiness Distribution Metric to assess link quality and this metric become an input parameter to improve the latency and reliability for Routing Protocol and Scheduling Algorithm for WSNs,

There are some studies to design a Routing Protocol in multi-hop low power networks such as RPL [2], LOADng [3], LRP [4] to seek effective routes. These protocols have to keep their own foot-print small and selecting the best way to route for data traffic. So this preliminary link quality measurement needs to be precise and accurate to choose the best route to sink node to achieve reliability target with a low-cost. However, the highly non-deterministic of the wireless link is a difficult challenge because of electromagnetic interference, and high attenuation, due to the presence of obstacles. It is necessary to deal with typical problems of wireless networks: burstiness link and interference. The interference link is the physical property of the environment that causes the packet transmission between different links to interfere with each other. Burstiness link is another physical property that means transmissions of the wireless link do not has a fixed probability of failure and have periods of continuous loss packet [5]. Due to these non-deterministic wireless links, it is difficult to offer reliability for packet transmit over the wireless networks.

The author in [5], [6] proposed mechanisms to solve the Burstiness link problem. Srinivasan et al. [6] presented a new metric called β to measure the link burstiness and showed that we can avoid link burstiness by having an inter-packet delay of 500 ms. But in the industrial area, 500ms of delay is a disadvantage for the network, we don’t have to wait that time to avoid the packet loss when link burstiness happen. Munir et al. [5] propose a new metric called maximum burst length (Bmax) as the maximum length of error burst, estimated by using empirical data for a long experiment. The novel routing algorithm called least-burst-route is used in conjunction with this scheduling algorithm to minimize the sum of worst-case burst lengths over all links in the route. However, the algorithm to calculate Bmax value is gathered from the experiment measure link with the long sequence of data-trace before settings the network. It makes this algorithm hard to apply to the ad-hoc network. And the use of Bmax is the maximum retransmission is overestimation for some kind of application.

This paper proposes a link quality estimation metric calculated by the receiver node the number of consecutive loss packet then makes the distribution of burstiness of each link. Our proposal can be easy to apply to the ad-hoc network. Moreover, we can use Burstiness Distribution Metric for the Routing Protocol to find the stable parent and for the Scheduling Algorithm to estimate the number of retransmission to reach a reliable target for the specific application. The remainder of this paper is organized as follows. In Section 2, we present some background and related works. The system model will be described in Section 3. In Section 4, we will show the evaluation result of our method and the conclusion will be given in Section 5.