Implementation of a centralized scheduling algorithm for IEEE 802.15.4e TSCH

Jules Pascal Gilles RUGAMBA1, Dinh.Loc MAI1, Myung.Kyun KIM1

1 University of Ulsan, 44610 Daehak-ro 93, Nam-Gu Ulsan, South Korea
mkkim84041@gmail.com

Abstract. IEEE 802.15.4e Time Slotted Channel Hopping (TSCH) is an important standard in the actual scalable Internet of Things (IoT), to habilitate deterministic low-power mesh networking. In TSCH, scheduling can be distributed or centralized scheduling. In this paper we describe the implementation of a centralized link scheduling for IEEE 802.15.4e TSCH based industrial Low Power Wireless Networks, known as Path Conflict free Least Laxity First (PC-LLF) algorithm in Contiki OS, a real time operating system for wireless sensor by using RPL protocol for network topology construction. PC-LLF prioritizes the assignment of the communications resources for each transmission. For each node to know its assigned cell in the schedules, we implemented a method to broadcast the schedule information packets to all transmissions generator (nodes) with the aim to reduce packets collision and loss. We compared our method with the broadcast of the same packet by Minimum Schedule Function (MSF) and the results show that our method performs better than the MSF and use less number of packets messages than other CoAP and OCARI broadcasting method. The implementation devices are Texas instruments CC 2650 Launchpad. The implementation aim is to make a real IoT network running a centralized scheduling algorithm of TSCH which can be effectively applied in the industrial networks and the results proved that.

Keywords: TSCH, IEEE 802.15.4e, scheduling algorithm implementation, Contiki, RPL

# Introduction

The use of Wireless Sensor Networks (WSNs) in industrial area is increasing and it is found in different industrial application such as monitoring applications, radiation check, leakage detection, distributed and process control [1]. Those real-time applications are sensitive to the delays and demand high communication reliability and a satisfaction of a high scalability. The end-to-end delay are constrained by upper bounds which varies according to the field of application [2]. To overcome those challenges, different wireless technology approaches which reduces the cost of wired installation, have been considered such as the IEEE802.15.4e with a MAC protocol named Time Slotted Channel Hopping (TSCH). With the time slotted and multi-channel frequency hopping features, TSCH reduces latency and ensures high resistance against interferences as well as multi path fading.

Although the IEEE802.15.4e standard defines the process to execute communication schedule, it does not define the process of how the schedule is built, maintained, updated as well as the design of the entity in charge of performing such tasks. Scheduling on TSCH networks has been studied widely. Two types can be distinguished: 1. Distributed scheduling where there is no central manager (or coordinator) to handle the schedules but nodes in the network agree on the schedule to be used; 2. Centralized scheduling where a scheduling is built by a central entity or Coordinator by collecting the network information. Our implementation will be based on a realistic centralized scheduling algorithm known as PC-LLF.

Path Conflict free Least Laxity First (PC-LLF) [3] is a centralized scheduling algorithm which assigns priority to each packet transmission dynamically based on its laxity (the remaining time to the end-to-end deadline) and the amount of latency imposed by the collisions throughout its path to the final destination node (root node).

With the aim to give a contribution to the study of implementation of centralized scheduling in TSCH networks in real environment, we implemented PC-LLF in real hardware, a low-power microcontroller of a Texas instruments TI CC2650 LaunchPad kit using Contiki OS an open-source implementation of a fully finished protocol stack based on Internet of Things standards. As the coordinator node makes the schedules to be used in a given network, the principal key during implementation of centralized scheduling, is the reception by all nodes in the network of the schedule table information packet i.e. each node has to know through that packet, at which timeslot and channel it has to wake up for transmission or reception. To avoid the loss of such essential packets, we designed a method to send it in broadcasting manner by assigning to each node its own timeslot to broadcast the packets, which eliminates competition between nodes, reduces collision and decrease considerably the loss.

The remainder of this paper is organized as follows. Some backgrounds are described in Section 2. A brief description of the PC-LLF will be given in Section 3. In Section 4 the implementation process will be described, in section 5 we will show the evaluation of our implementation and the conclusion will be given in Section 6.

# Background

## IEEE 802.15.4e (TSCH) and RPL

TSCH builds a globally synchronized mesh network where nodes can join the network after hearing a beacon from another node. The time synchronization flows from the PAN coordinator to the leaf nodes along the network.

In TSCH the time is divided into a fixed number and equal-sized timeslots N which form a repeatable and periodic slotframe. One timeslot is long enough to contain the transmission of a data frame and its acknowledgment. Based on a schedule, a node is set in transmitting, reception or sleeping mode at a specific timeslot. TSCH defines the Absolute Slot Number (ASN) which is the total number of timeslots that has elapsed from the start of the network or a randomly determined start time by the coordinator.

It is used as the slot counter and it increments globally in the network on the beginning of each time slot.

In TSCH, 16 or less different channels can be used for communication. Multiple nodes can communicate at same time slots using different channel offset. The communication between 2 nodes in TSCH occurs in cell which is a pair of a timeslot in the slotframe and channel offset in that timeslot. A communication cell can be shared or dedicated i.e. contention-based with CSMA back-off or contention-free respectively. In the timeslot, the sender begins the transmission TsTxOffset after the beginning of timeslot, and the receiver listens to the channel guardTime before. Thereby, it implies that the devices cannot be desynchronized more than guardtime μs. In this paper, the implemented centralized scheduling algorithm allows only the use of dedicated cells.

To form a mesh network or any topology in TSCH, there exist different used protocol such as RPL[4] which is a routing protocol for constrained Low power and Lossy network standardized by the IETF group and designed to meet some requirements described in [5] such as latency, reliability, scalability among others. The routers are organized through the destination oriented directed acyclic graph (DODAG) rooted at the edge router. It is used to get an accurate vision of the network status and thus to compute the best path for downward and upward traffic. Those path are computed based on an Objective Function (OF) and its set of metrics and constraints.

Designed for IPv6 networks, RPL uses the features of ICMP6 neighbor discovery messages for its auto-configuration.

The building process of a DODAG is triggered by a root node and four types of control message DODAG Information Object (DIO), DODAG Information Solicitation (DIS), Destination Advertisement Object (DAO), Destination Advertisement Object Acknowledgment (DAO-ACK) are used between nodes during that process to exchange multiple information.

## Contiki OS

Written in C, Contiki operating system [6] was designed for resource constrained devices like Low-Power wireless IoT devices and includes in its latest version [7] the implementation of TSCH based on the IEEE 802.15.4e standards. Its library has a full TCP/IP stack for radio network communication and it supports different variety of radio enabled platforms. Contiki network stack includes diverse protocol at each layer and some modules are specific to each protocol and some are common modules. The queuebuf.c module which is common for example provides a way to manage multiple packets at a time. Each other module to access the queuebuf.c needs to maintain pointers to it. 6LoWPAN at adaptation layer would take advantage of queuebuf.c for fragmenting IPv6 datagrams into multiple packets and TSCH at MAC layer use it for queues transmission. From IETF (Internet Engineering Task Force) known networks and applications standards, Contiki supports in its library TCP, 6LowPAN, UDP, RPL, IPv6 and CoAP. Contiki OS includes an event driven kernel used to load and unload process at run time.

# PC-LLF: Path Conflict Aware Least Laxity First Algorithm

The PC-LLF algorithm [3] is a centralized algorithm designed over IEEE 802.15.4e TSCH with the aim to reduce latency of messages by finding feasible schedule for all transmissions of each flow along their designated path. As described previously, the transmission in TSCH occurs at a certain time slot t and channel ch, which compose a cell (t, ch).

Designed for TSCH networks, PC-LLF algorithm obeys to some constraints of that technology described below. Firstly, network forms a topology mesh network (Fig.1.) as a Directed Acyclic Graph (DAG) G = (N, L) where N= {N1, N2, … , Nn} represents all devices in the considered TSCH networks and arcs in L are communication link. NpNq represents a link where Np is the transmitter node and Nq is the receiver node. Interfering links cannot be scheduled on the same cell and the duplex conflicting links cannot share the same time slots. The duplex conflicting links are all the links that have same receiver or sender nodes. From the Fig.1, duplex conflicting links of the link N6N2 are CNF (N6N2) = {N5N2, N4N2, N2N1}.

Fig. 1. Example of network G = (N, L), N= {N1… N11} and D the Period assigned to each transmission from its source node; N1 is the root node

All transmissions are upward and considered periodic. The transmission’s period which is equal to its deadline, is chosen randomly by each node from a power of two value [23, 29]. A packet’s transmission miss deadline when it arrives to the destination later than its release time added its deadline considered in time slot value. To calculate priority used by PC-LLF to make feasible schedule, some parameters are considered such as time window denoted TW[x, y] for each transmission which is the time phase from the earliest time slot (x) and the latest time slot (y) that the transmission can occur. The earliest time slot is the actual time slot and thus it is dynamically recomputed at each timeslot. The latest time slot has a fixed value and it depends on the deadline value. The second considered parameter is the average delay due to collisions that each remaining transmissions may deal with at the time slot S. The considered collisions are duplex conflicting collisions which happen due to a transmission existing in a conflicting link with a time window TW that overlaps with the time window of the actual considered transmission. Let denote the amount of conflict transmissions with a certain transmission Ti as NCFL (Ti). The next step is to get the average latency of a packet due to the collisions through the remaining path at a certain time slot.

The remaining path is considered to be composed by the actual transmission and the future transmission of the packet in order to reach the root nodes. Let denote the average latency due to duplex conflicting transmission at a certain time slot S as DCFLavg (Ti, S) and its value change dynamically at each time slot because the amount of duplex conflicting transmissions vary due to the fact that some transmissions may be scheduled previously or others may be released later. Let denote k, the number of hop count away for a transmission to reach the root node. Thus the average latency is given by the sum of all conflicting links divided by the number of hop count as in the equation below:

DCFLavg (Ti, S) = ∑ NCFL (Ti)/k	(1)

From the equation (1), the priority of a transmission at a certain time slot S is computed by the difference between the time window of the packet’s transmission and its average latency as described in the equation below:

Pr (Ti, S) = TW[x, y] - DCFLavg (Ti, S)	(2)

From equation (2), the lowest value corresponds to a higher priority. Based on the computed priorities for all transmissions, PC-LLF algorithm assigns the time slot and channel to the transmission with the lowest value. When several transmissions have same priority, the transmission with the smallest length of time window will be assigned firstly. Furthermore, if the transmissions are executed in an environment with more than one channel, among the transmissions which are not in duplex conflict with the firstly assigned transmission, the transmission with highest priority is assigned to a different channel in the same time slot.

# Implementation

The PC-LLF algorithm was implemented using Contiki OS into the low-power microcontroller of a Texas instruments TI CC2650 LaunchPad kit. Its microcontroller is based on ARM Cortex-M3 architecture which operates at 48MHz, with 128KB of flash memory and 20KB of SRAM.

Fig. 2. Implementation process

Fig.2 gives a big picture of the implementation process divided into four parts. Firstly is the tree construction where nodes make a network topology using the RPL routing protocol. Secondly is the execution by root node of the implemented PC-LLF algorithm. At third, root node assigns to each node a timeslot to broadcast the schedule table information packet, broadcasts the resultant schedule table to all nodes and that part includes also the broadcast of the schedule table information packets. Fourthly all nodes will start data transmission following the PC-LLF cells assignment.

## Tree Construction

Contiki OS includes the RPL routing protocol in storing and non-storing mode. We chose to use RPL non-storing mode as it allows the root node to collect all network information from each node in the network through the exchanged packets. We assume a fix number of nodes in network and known by root node in the beginning and thus, it can check through defined function if it had received all the information from all nodes in order to start running the PC-LLF algorithm

The nodes form a network by RPL non-storing mode and run Minimum Scheduling Function [8] in the tree construction period. The root nodes broadcast first the DIO packet to inform of the existence of a network and each receiver node try to join by following the process describes in [4]. Once joined, the node may also act as a router and broadcast the DIO at its turn.

| Type: 0x0A | Option length | Period |
| --- | --- | --- |
| 1 byte | 2 bytes | 2 bytes |

Fig. 3. Structure of new Period Option in RPL DAO packet

There exist different DAO option used for carrying routing information such as target option which carry the information of DAGid of the new joiner node and the transit option which carry multiple information such as the joiner node’s parent information and thus the links information are forwarded to the root node. To send to the root node the period value of each node’s transmission needed to run PC-LLF, a new option named Period Option as shown in Fig.3 was added in the DAO packet by modifying the rpl-icmp6.c module in Contiki OS. The period option has three field: Type to identify the option type, Option length to specify the length and Period which contains the period for the transmission.

## Implementation of PC-LLF Algorithm

From the link and transmission’s period information collected by root node during the tree construction period, the root node executes the PC-LLF algorithm to schedule all node’s transmissions. In case of an unschedulable network topology, the root node has to stop the execution and the change in the topology is required.

The input parameter for the execution of algorithm are G the links information built from RPL previously, R which is the list of released transmission and in the beginning each node is considered to have a transmission to be forwarded. The next parameter is P, each node’s transmission period which is equal to its deadline and Nch the number of channel used in data transmission and we considered two channel (Nch = 2). The output is the schedule table of all transmissions (Fig.4) which is forwarded to all nodes from root node.

The PC-LLF algorithm was written in C as it has to run under Contiki OS and was added as new created library in Contiki. The library access the routing layer handled by RPL to get network topology with routing links and period of each transmission as input.

Fig. 4. Example of scheduling table information from a network topology in Fig. 1

The algorithm must finish to schedule all transmissions no longer than the Least Common Multiplier (LCM=16 in the example) of all periods value in the network as seen in example Fig4. At timeslot one (Ts=1, ch=0), the transmission from node 2 to root node (1) was assigned. The transmission are noted as T (Nid – Mid) – I where Nid is the source node id of the packet, Mid is the receiver node id and I is the instance of the transmission which specifies the current period of the transmission, as all transmission are periodic. In the example (Fig.1) nodes N2, N3, N8, N4, N6 and N9 for example have two period of transmission within the LCM. The number of period of transmission is defined by the LCM divided by the period value of the sender node’s transmission.

## Assignment of timeslot to broadcast the schedule table information

This process is divided into three parts. Firstly, we assigned timeslot for each node to broadcast the schedule table information and secondly is to send the assignment information to all nodes. Last one is the broadcast of schedule table information packets by all nodes as assigned.

## Assignment of timeslot to broadcast the schedule table information

After execution of PC-LLF algorithm, each node in the network has to receive the schedule information. For that, the packets have to flow from the root node towards all the nodes in the network. To ensure the minimum loss of schedule table information packets, we assigned a dedicated timeslot to each node for broadcasting that packet.

Based on the constructed network topology and the gathered network information by root node, the assignment was done by Breadth-First Search Algorithm (BFS) [9]. Each node in the network is assigned a unique identifier (node_id) used to assign timeslot to the nodes to broadcast the schedule table information packet. By following the BFS standard, we assign firstly the master node (root node) in the first timeslot and then the algorithm traverse the tree from the adjacent node of the root node and assigns timeslot to the node with the smallest node_id value until all the adjacent nodes to the root nodes are assigned. At next hop-count, the algorithm firstly traverses the adjacent nodes to the firstly assigned nodes from the adjacent nodes to the root and assign timeslot based on the value of node_id. The process continues until all nodes are assigned. The result of assignment is shown in Fig. 5.

Fig. 5. Result of assignment of Timeslot to each node in Fig.1 to send schedule table information

This assignment reduces the number of collision firstly when transmitting as only one node is assigned for transmitting and there is no competition between nodes, and secondly when receiving the packet because one node (or more) can receive only from one transmitter, the assigned node.

## Sending the assignment information to all nodes

The assigned timeslots must be known by all node in the network and thus the root node will, after creating the assignment information packet, broadcast it to each node. To realize that, we used the Contiki OS implemented module named tsch-queue which defines a queue of outgoing packets for each neighbor and we added others queues neighbor beside the existing one, for the purpose of our implementation. The n_assign is used for queuing the assignment information packet when it is broadcasted and n_data for handling the outgoing data packet used specifically for data transmission period.

Fig. 6. Packet frame format of assignment information packet

The root node triggers the download of assignment information packet by broadcasting the packet. When a node receive the assignment packet, it activates its assignment information download period. Each node is assigned a number of timeslot delay between two transmissions. A larger number of node in the network implies a large packet and hence it has to be divided into parts to be transmitted. Each assignment information packet in node’s queue is transmitted after the delay. The use of MSF i.e. one timeslot and one channel offset as specified in the standard during that period may cause collisions in the packet traffic. To reduce the collision, we implemented a competition scheme within a timeslot which allow each node to access the communications resources by reducing contention time based on the sequence of the assignment information packets received and by checking if the channel is idle or not.

When broadcasting the assignment information packet, we took in consideration the maximum packet size originally support in TSCH which is 127kbytes. Fig.6 shows the packet frame format and the size we used for the assignment information packet. Based on the frame format, one packet can include up to 30 assignment information which make a total size of 118kbytes.  This allow to send a large number of assignment information packet while the allowed maximum size of packet is not exceeded.

Taking into consideration that the assignment information sent in one packet are considerable and that one assignment information contains only one node information, we assumed that the needed number of timeslot to send all packets in a given network and to be received by all nodes is equal to the number of nodes in the network multiply by two to avoid loss.

## Broadcast of the schedule table information.

To transmit data packets, each node needs to know in which timeslot it has to wake up for transmitting or receiving the data packet. That means the broadcast of the result of the PC-LLF algorithm scheduling (Fig.4) which is done by following the assigned timeslot for each node to broadcast the schedule information packet. After the reception of the schedule information packet, the node adds it to the queue via n_schedule queue neighbor used for queuing the schedule table information packet and created through the tsch_queue module described above.

Fig. 7. Schedule information packet frame format

The node will then process the received packet by storing in RxList the information of its reception cells and those for transmitting cells are stored in TxList. After processing the packet, the node get the packet from the queue and broadcast at its turn in its assigned timeslot.

Fig.7 shows the schedule information packet constitutes by MAC header, packet type to identify the packet. A large number of nodes in the network implies the fragmentation of the full schedule information packet and thus the payload is divided into different fields to handle that. It has LCM which is the least common multiplier for all period value of all transmission, Number of part which contains the number of packets to transmit all the schedule information packet, the current part number for identifying the currently being transmitted packet, length with the value of total transmission information in the current packet, sched info containing the assigned cell for a transmission as well as the sender and receiver node address. It contains also the curr Ts which informs on the current used timeslot and the Download Ts duration which is the number of timeslot to be used for broadcasting the schedule information packet. Considering that each node has its own timeslot to broadcast the packet, we considered the maximum duration to broadcast the packet to be equal to the number of node multiply the number of packet used to transmit all the full packet (Number of Part). Based on the packet size and the allowed size packet to be transmitted in TSCH (127kbytes), one packet can contain at most 13 scheduled transmission information (or cells) which make the size equal to 121kbytes.

Fig. 8. Reception delay in timeslot value of the schedule table information packet

## Data Transmission period

During the data transmission period, all nodes transmit their data towards to the root node based on the schedule computed by the PC-LLF algorithm. After the broadcast process, each node knows from the RxList and TxList exactly at which cell it has to transmit or to receive data packet. The Absolute slot Number (ASN) described above, gives the possibility to know exactly the current used timeslot and hence, each node can trace and know its timeslot to wake up.

The data transmission period duration or number of timeslot for data transmission is equal to the LCM value of all node’s periods in the network. During data transmission, when a node wakes up at its associated cell, it checks first if the current cell is for reception mode then the node listen for the packet and after reception, it adds the packet in its queue and if is for transmission mode, the node firstly checks if any packet is available in its queue buffer, get the packet from the queue and send it; after transmission the packet is removed from the queue. In case the queue buffer is empty, it switches into sleep mode. Once the packet is ready, the 6LoWPAN of Adaptation Layer, passes it to the MAC layer for being transmitted. Likewise, when receiving packet, the MAC layer passes it to adaptation Layer using the packetbuf.c module. As each node wake up at the specific cell for transmitting or receiving, the data packet transmission is done by broadcasting from its source node and only the node which woke up at that cell for reception can receive the packet.

# Implementation evaluation

The PC-LLF algorithm was implemented using Contiki OS real hardware of Texas Instruments TI CC2650 LaunchPad kit in which one acts as root node.

We compared the schedule table information packet reception delay in the network when the packet is broadcasted by Minimum scheduling function (MSF) and when it is broadcasted by our assigned broadcast method described in section 4.3 by considering the number of timeslot used in relation to the number of nodes in the network. The more nodes are in network, the more packets of schedule information as the full packet is fragmented into multiple packet to avoid oversizing the maximum allowed packet size. We also compared the number of packets messages used by our method and those used by CoAP and OCARI to broadcast the schedule packets as described in [10].

From Fig.8, we can see the variation of the reception delay of the schedule table information packet when broadcasted by MSF and when our assigned broadcast is used. We made a test using firstly 5 nodes and then 10 nodes, 15nodes and 20nodes in the network. The result show that with the short number of node in the network, the broadcast using MSF uses less timeslot than our assigned broadcast. This is due to less competition process as there are only few nodes in the network. As the number of nodes increases, we can see a considerable difference and the assigned broadcast outperforms the MSF broadcast as the competition becomes though when then number of node increase and collisions are more frequent which can cause considerable loss and make MSF vulnerable compare to our assigned broadcast.

Table 1. Table of each Flow and their period and delay time.

| Flow | Period | End-to-end Delay | Flow | Period | End-to-End Delay |
| --- | --- | --- | --- | --- | --- |
| 2 1 | 8 | 1 | 7 1 | 16 | 8 |
| 3 1 | 8 | 4 | 8 1 | 8 | 6 |
| 4 1 | 8 | 3 | 9 1 | 8 | 2 |
| 5 1 | 16 | 14 | 10 1 | 16 | 9 |
| 6 1 | 8 | 5 | 11 1 | 16 | 7 |

The period of each transmission equals to its deadline and that value is expressed in timeslot number. The end-to-end delay of each transmission flow (represented in first column with sender node id  destination node id), expressed in timeslot number as shown by Table1, is less than its period value and thus all the flow didn’t exceed the deadline for transmission.

Table 2. Number of packets required to broadcast the schedule packets.

| Number of node | Number of cells | Our method (packets) | CoAP (packets) | OCARI (packets) |
| --- | --- | --- | --- | --- |
| 5 | 12x7(84 bytes) | 1x5 | 3x5 | 2x5 |
| 10 | 28x7(196 bytes) | 3x5 | 7x5 | 3x5 |
| 15 | 60x7(420 bytes) | 5x5 | 14x5 | 6x5 |
| 20 | 78x7(546 bytes) | 6x5 | 18x5 | 7x5 |

In Table 2, the number of cells column describes the existing cells in one schedule table and obviously increase when the number of nodes increases. Each cell is 7 bytes as shown in Fig.7. Our method allows at most 91 bytes over the 127 bytes max packet as the remaining is used for other system and configuration information.

As described in [10], CoAP and OCARI allow respectively 32 bytes and 80 bytes. Considering the topology in Fig.1, the nodes that can broadcast the schedule packets i.e. with at least one children node, are 5 and thus we multiply by 5 to get the total number of packets to broadcast. As result, our method use less packets which has a positive impact on node’s duty cycle as well as on the duration of broadcasting the schedule.

# Conclusion

In this paper, a centralized schedule for TSCH was implemented using Contiki OS and RPL for the network topology. The PC-LLF component was integrated in the existing stack structure of Contiki OS. To ensure the reception by all nodes of the schedules table information, we successfully used our assigned broadcast method which shown better results than broadcasting with Minimum Scheduling Function (MSF) algorithm and use less packet than CoAP or OCARI. The result of implementation show that our method to broadcast  schedule table and the PC-LLF algorithm can run in real device which is helpful for an implementation in real environment with a centralized schedule.

# Acknowledgements

This work was supported by the 2019 Research Fund of University of Ulsan.

# References

- B. Sinopoli, C. Sharp, L. Schenato, S. Schaffert, and S. S. Sastry, “Distributed control applications within sensor networks,” Proc. IEEE, vol. 91, no. 8, pp. 1235–1246, Aug. 2003

- R. Zurawski, “Networked embedded systems: An overview,” in Networked Embedded Systems, R. Zurawski, Ed. Boca Raton, FL: CRC Press, 2009, ch. 1, pp. 1.11–1.16.

- A. Darbandi and M. K. Kim, “Path Collision-aware Real-time Link Scheduling for TSCH Wireless Networks”, KSII TRANSACTIONS ON INTERNET AND INFORMATION SYSTEMS, accepted to be published, 2019.

- T. Winter, P.Thubert, A. Brandt, J.Hui, R. Kelsey, RPL: IPV6 “Routing protocol for Low Power and Lossy Networks”, IETF request for comments 6550, March 2012

- E.K. Pister, E.P. Thubert, S. Dwars, T. Phinney, “Industrial routing requirements in Low-Power and Lossy Networks”, IETF request for comments:5673, March 2009

- A. Dunkels, B. Gronvall, and T. Voigt. Contiki - a lightweight and flexible operating system for tiny networked sensors. 29th Annual IEEE international conference on Local Computer Networks, pages 455{462, November 2004. 17

- Contiki 3.x. https://github.com/contiki-os/contiki.Online; accessed 2-June-2016. 17

- Minimum Scheduling Function (MSF) https://tools.ietf.org/html/draft-ietf-6tisch-minimal-21

- Breadth-First Searching(BFS) Algorithm https://en.wikipedia.org/wiki/Breadth-first_search

- Erwan Livolant, Pascale Minet, Thomas Watteyne, “The Cost of Installing a 6TiSCH Schedule” AdHoc-Now 2016 - International Conference on Ad Hoc Networks and Wireless, July 2016, Lille, France